Landowner Report

Station Information

Station ID	2-RED003.65

Stream Name	Reed Creek

Description	Downstream of Rt. 637

Year		2004, 2011, 2013, 2015

Regional Office	BRRO

County		Bedford

Basin		James

Photo of sampling station, Spring 2015

Data Results

Spring		Fall		Spring		Fall		Spring		Fall		Spring		Fall	Status

2004		2004		2011		2011		2013		2013		2015		2015

Temperature (C/F)	10.3		18.1		18.0/64		13.2/55		16.1/61		15.6/60		9.4/49		16.8/62	Optimal

Dissolved Oxygen (mg/l)	12.3		9.7		10.0		10.4		10.1		9.6		11.4		9.6	Optimal

pH			6.4		6.2		7.5		6.1		7.4		7.6		6.9		7.5	Optimal

Conductivity (uS)		34		45		30		61		35		54		34		54	Optimal

Bacteria (cfu/100ml)	140		na		160		na		380		na		98		na	Optimal

Total Nitrogen (mg/l)	0.54		na		0.56		0.51		0.56		0.34		0.46		0.51	Optimal

Spring		Fall		Spring		Fall		Spring		Fall		Spring		Fall	Status

2004		2004		2011		2011		2013		2013		2015		2015

Total Phosphorus (mg/l)	0.03		0.04		0.03		0.02		0.05		0.02		0.03		0.04	Optimal

Metals Index (ccu)	0.26		na		0.23		na		0.21		na		0.20		na	Optimal

Biological Index (0-100)	67.7		53.7		61.6		71.0		70.6		64.3		71.1		68.5	Optimal

Habitat Index (0-200)	175		158		171		152		175		156		164		152	Optimal

Biologist Comments:

This site was sampled as a part of the Probabilistic Monitoring Program in 2004, 2011, 2013 and 2015.  The station was in a remote section of Bedford County several miles upstream of the confluence with the James River.  Biomonitoring surveys of aquatic macroinvertebrates resulted in good numbers of pollution-sensitive taxa such as mayflies, stoneflies, and caddisflies.  Scores for habitat surveys were in the optimal range with a slight impact from sediment deposition.   Fish community sampling resulted in a mix of suckers (torrent, white, and northern hog suckers, golden redhorse), minnows (bluehead chubs, central stonerollers, common shiners, crescent shiners, blacknose dace, fallfish, longnose dace, and rosyside dace), darters (fantail, johnny and stripeback ), blueridge sculpins, margined madtoms and a few game species such as smallmouth bass, rock bass, redbreast sunfish.

Larval stonefly “indicator of good water quality”

Biologist Contact Information:

George Devlin, Regional Biologist

VDEQ- Blue Ridge Regional Office

3019 Peters Creek Road

Roanoke, VA 24109

Phone: 540-562-6826

Email:  george.devlin@deq.virginia.gov

The 'Landowner Report’ is not an official site specific VDEQ water quality assessment.  This report summarizes water quality findings based on a limited sampling event.  The primary application of probabilistic monitoring data is to establish regional/statewide water quality conditions and not to perform a site specific characterization.  Please see the enclosed insert for more information.