Landowner Report

Station Information

Station ID	4AXOH000.06

Stream Name	Unnamed Tributary to South Fork Goose Creek

Description	Off of Audrey Drive near Montvale

Year		2014

Regional Office	BRRO-Roanoke

County		Bedford

Basin		Roanoke								Photo of sampling station

Data Results

Spring	Fall	Status

Temperature(C/F)	6.9	14.5	Optimal

Dissolved Oxygen(mg/l)	11.7	9.9	Optimal

pH			7.3	7.7	Optimal

Conductivity (uS)		32	40	Optimal

Bacteria	(cfu/100 ml)	180		Optimal

Total Nitrogen (mg/l)	0.08	0.32	Optimal

Total Phosphorus (mg/l)	0.07	0.13	Optimal

Metals Index (ccu)	0.05		Optimal 					Approximate location of station

Biological Index (0-100)	74.1	71.0	Optimal

Habitat Index (0-200)	167	141	Optimal

The 'Landowner Report’ is not an official site specific VDEQ water quality assessment.  This report summarizes water quality findings based on a limited sampling event.  The primary application of probabilistic monitoring data is to establish regional/statewide water quality conditions and not to perform a site specific characterization.  Please see the enclosed insert for more information.

Biologist Comments:

This site was sampled as a part of the Probabilistic Monitoring Program in 2014.  The site was in a small headwater stream that had good numbers of mayflies in the spring and stoneflies in the fall.  Overall, the benthic community had good diversity and high numbers of pollution-sensitive taxa. Most habitat scores were good except for sediment deposition in the fall when the stream was at low flow conditions.

Larval stonefly “indicator of good water quality”

Biologist Contact Information:

George Devlin

Regional Biologist

VDEQ  BRRO-Roanoke

3019 Peters Creek Road

Roanoke, VA 24109

Phone: 540-562-6826

Email:  george.devlin@deq.virginia.gov